Brass2Go

DOCUMENTATION + USER MANUAL

Authors:

Usman Zia

Luke Garland

Toshiro Taperek

Developed for:

ILS – ENEL 300

Schulich School of Engineering

April 17, 2019

Table of Contents

1.  Introduction                                                                                                                    3

1.1. Project Introduction

1.2. Key Stakeholders

1.3. Target Market

1.4. Persona Description

1.5. Concept Description

1.6.Fundamental Project Requirements

2.  Product Constraints                                                                                                      8

2.1. Design Limitations

2.2. Technical Limitations

2.3. Financial Budget

3.  Product Design                                                                                                             10

3.1. Circuit Design

3.2. Hardware/Software Prerequisites

3.3. PIC Programming

4.  Product Development Cycles                                                                                    18

5.  Project Management                                                                                                  19

6.  Validation and Testing                                                                                                22

6.1. Hardware Testing

6.2. Software Testing

7.  Appendices                                                                                                                    23

7.1. References

7.2. User Manual

1.   Introduction

1.1. Project Introduction

Brass2Go is the result of the final project outlined in ENEL 300, a 2-year Electrical Engineering Course offered by the Schulich School of Engineering. This final project will aid in building a practical foundation in the listed courses taken in the Winter Semester of 2019:

•   ENEL 343 – Circuits II

•   ENEL 327 – Signals and Transforms

•   ENEL 361 – Electronics

•   ENEL 369 – Computer Organization

This final project has been assigned on the day of Jan 07th, 2019 and demonstration deadline is on the day of April 18th, 2019. The project requirements have been set by instructors for the above courses and Dr. Laleh Behjat.

1.2. Key Stakeholders

The key stakeholders of this final project have been listed below:

-     Schulich School of Engineering

-     Integrated Learning Stream – Pilot Program

-     Faculty of Electrical and Computer Engineering

-     Instructors of the courses listed in section 1.1.

-     Industry Experts and Judges

1.3. Target Market

The Target Market for “Brass2Go” will not be limited to professional brass instrument musicians but also for hobbyist musicians. The music industry has been significantly growing within the past few years peaking at 5.9% in 2016, the highest to date, with revenues capping at

$15.7 Billion (USD) [4]. Our product “Brass2Go” is a highly innovative learning tool with error detection algorithms making it a new entry into the market. Given the competitive industry, our product will have an easier market takeover as this will be the first in the market.

1.4. Persona Description

The “Brass2Go” has been created for the 90% of talented musicians that are not even known by the industry and are working part time jobs just to keep funding their music careers. Thus, we have had affordability in mind and created “Brass2Go” with low cost technical parts.

The following is a common persona for our product:

A talented university student that dreams of joining a professional trumpet band and showcase her art of music. However, to accomplish her dream she juggles commitments between her education and practicing her trumpet. She believes that hard work and determination is the key to success and desires to find innovative methods of practicing her instruments while commuting back and forth to school.

The following is how the common persona intends on using:

She is on her way to school that consists of an hour-long C-train commute. As an opportunistic person she desires to practice her trumpet which is not possible due to the large size and respect for others on the train. She decides on investing in a portable music player but is unable to improve her skills as no insights are available. Brass2Go provides her with the opportunity to practice on the train everyday to school resulting in 240 more hours of practice per academic semester. With our music software connected with Brass2Go she is now able to spot her mistakes and make improvements and is on her way to a rising trumpet prodigy!

The following is the description from one of our valued users –  Salah Sheikh:

“This idea is great as I am always looking for learning tools to improve my muscle memory. Also, I was shocked that it could also tell me how many mistakes I made and my accuracy rating! This I believe would definitely help and I can keep doing it while on commute!”

USER FEEDBACK CATALOGUE:

5

1.5. Concept Description

Brass2Go is a portable learning tool which allows a musician to practice the rhythm of a song on the go and hence excel at his performance by data driven insights. Unlike other instruments

that provide no insights for improvements in a song and are not portable.

The idea behind Brass2Go is for the 90% of the unknown musicians that work jobs and need more time to practice their skills and compete with the 1% of successful musicians with significantly higher access to resources. Brass2Go will aid in providing the necessary instrument functionalities with error detection algorithms for feedback resulting in major improvements

for its users.

The inspiration behind the concept is for music fanatics to have access to a broader range of musicians thus adding creativity to the music industry currently dominated by a few selected individuals.

1.6. Fundamental Project Requirements

The “Brass2Go” has been developed under the strict requirements of ILS 2019 project guidelines:

-	The financial budget allocated for the project should not under any circumstances exceed $60 (CAD).

-     The project must be based on the PIC16F1778 Microprocessor.

-     The end project must have the ability to play audio.

2.   Product Constraints

2.1. Design Limitations

The following are self-defined design constraints in order to provide basic functionalities. These design constraints were imperative during the development of Brass2Go as the central idea always had to take these limitations into consideration.

Table 1: Self-defined design constraints of Brass2Go

Design Constraints – Description

1          Programming onto the PIC16F1778 Microcontroller

2          Speaker and Head-phone output provided

3          Must be able to output song accuracy

2.2. Technical Limitations

The Technical Constraints for this final project was outlined in the ENEL 300 – Project Rubric and the following table summarizes key constraints that directly affects the development of Brass2Go.

Table 2: Technical Constraints of Brass2Go

Technical Constraints - Description

1          Implementation of the PIC16F1778

2          Must demonstrate integration of ILS Courses

3          Discrete Components

4          Designed and developed by the assigned team

5          Assembled by the assigned team

2.3. Financial Budget

The Financial Budget for this final project was outlined in the ENEL 300 – Project Rubric and the following table summarizes the financial budget for the development of Brass2Go.

Table 3: Financial Budget for Winter 2019 for the duration of 4 months starting January 07th,

2019 –  April 18th, 2019.

Our goal for affordability was achieved as the total cost of parts for this project returned $16.60 (CAD) calculated on April 17th, 2019.

3.   Product Design

Product Function Description

Brass2Go is a portable device that allows brass musicians to practice their instruments and compositions on-the-go. The device, powered by a PIC16F1778 microcontroller, plays an audio (.wav) file from a microSD card and the user plays the finger sequence along with the song. If the user plays the note incorrectly (i.e. the wrong note and/or at the wrong time) a red LED will light up. In contrast, a green LED will light up if the note is played correctly. Once the file is finished playing, an LCD screen will display the number of notes played incorrectly, and the percent of notes played correctly (e.g. 5/26 wrong -- 80.7% correct). During playback, the user can pause and play.

3.1. Circuit Design

Table 4: Circuit Schematic of Brass2Go featuring the PIC Microcontroller

3.2. Hardware/Software Prerequisites

The hardware used for this device is:

•    PIC16F1778 Microcontroller

•    2-line LCD Display (Midas MC21605G12W‐VNMLWS)

•    SPI SD Card Reader (AT25SF321)

•    ENEL 343, Circuits II, Audio Amplifier and Filter (TLV4112 Op Amp)

•    Red and Green 20mA LEDs

•    Switches (MJTP Series)

•    3.5 mm Headphone Adapter

•    Resistors (10kΩ for switches, 22Ω for LEDs, 10kΩ potentiometer and 960Ω for

volume knob)

CAD Designed Trumpet Valves emulating realistic trumpet valve:

Software Prerequisites:

The following software must be installed on your Windows, MacOS, or Linux machine:

•   MPLAB X

•   XC8 Compiler

•   Python 3

•   Mido (pip package)

•   HxD

•   MuseScore (optional)

•   Audacity (optional)

MPLAB X and XC8 Compiler

Once installed, you can then compile and program the PIC16F1778 microcontroller with the

Brass2Go.X project. This only needs to be done once.

Mido

Mido is a python package that is used by formatter.py to parse MIDI files. Most installations of Python 3 include pip (the Python package installer), but in the event pip is not a recognized command.

HxD

HxD is used to load the formatted .wav file to the SD card.

MuseScore and Audacity

These programs are used to generate the required audio files for formatter.py. If you already have MIDI and .wav files for your composition, these programs are not needed. MuseScore is a convenient open-source platform to convert sheet music and MusicXML files to MIDI and audio (.wav) files. Ensure you are using the correct key, and instrument type in MuseScore when you are generating the MIDI and audio files.

Audacity is an open-source platform to edit and mix audio files. Use Audacity to ensure that the audio (.wav) files that you are using are:

•   Mono

•   16-bit PCM

•   Sample Rate (Project Rate) = 44100 Hz

3.3. PIC Programming

•   To access all of the code please visit:  https://github.com/usmanziak/Brass2Go

1.   The following code snippet explains the workings of the formatter.py script:

This python script is designed to encode the correct button presses into the .wav file (audio file)

so we can play the audio file from the SD Card with encoding information.

The above code will check if the button should be pressed or not and set the Lowest Significant Bit to 1 if the button should be pressed and vice versa for setting the LSB to 0 if button not pressed.

2.   The following code snippet explains the timer functions within the PIC

As shown in the description above this header file will set the system clock speed to 32 MHz and Configure Timer2.

3.   The following code shows outputting data onto the LCD.

The above code “LCD_Write” returns an output onto the PIC16F1778 Microcontroller.

4.   The following code snippet explains how the SD card reads and outputs the encoded

.wav file.

The above code displays how we coded to interact with the SD Card

4.   Product Development Cycles

Alpha Phase:

The idea behind the project was to create a learning tool with accuracy output for brass instrument players in order to give them the opportunity to practice their skills on the go. As suggested during the Alpha Phase of the project we launched a basic functional prototype but found several issues with running the LCD, input from the keys and our accuracy algorithm at the same time given limitations with the PIC microcontroller.

Beta Phase:

During the Beta Phase we had to change fundamental functionality of Brass2Go, from the results of our Alpha Phase, the most salient being that we decided on not providing LCD output during the song. However, we successfully tested displaying an accuracy percentage after every song played.

Pilot Phase:

Finally, in the Pilot Phase our group focused on our persona and improved the usability of

Brass2Go by creating a portable final product of Brass2Go with features such as:

1.   Battery Powered – To make it easier to use on the road.

2.   Head Phone Jack – For our persona to play Brass2Go in public.

3.   Improved keys – Adds in to the high-quality “feeling” of our product.

5.   Project Management

The development process for Brass2Go was heavily based on the Agile Methodologies:

-	In Agile project management, testing is integrated during the cycle, enabling our group to make immediate changes as issues arise. This management technique has significantly improved our execution of Brass2Go.

-	Incorporating daily testing and continuous integration allowed faster addressal of issues and in time as the knowledge of Brass2Go was as relevant as possible.

-	Moreover, Agile technique also compromises of conducting sprint retrospectives resulting in continuous improvements and increased project control with team transparency.

-	Overall, Agile Project Management in ENEL 300 gave us the opportunity to learn about effective project execution with several development cycles and how to incorporate continuous testing.

Attached is a copy of the second sprint –  Backlog and Burndown chart for our evidence of efficient team management to deliver this project:

21

6.   Validation and Testing

6.1. Hardware Testing

Hardware Testing compromises of continuity testing which essentially tests for solder bridges and other possible issues on the designed perf boards. Moreover, the oscilloscope is a great tool for further testing the output at several connections and specifically the output from the PIC headers.

6.2. Software Testing

We integrated Agile development into our software development process. That includes continuous integration, basically testing your software at every branch merge on Git.

This allowed us to check for errors at every integration by a team member on the master branch. The following diagram displays how Agile Methodologies tie in together:

[6]

7.   Appendices

7.1. References

[1] K. Murari, ‘ENEL 361 Professor’, University of Calgary, 2019.

[2] J. Long, ‘Teaching Assistant’, University of Calgary, 2019. [3] PIC16F1778 Microcontroller, microchip. [online] Available: http://ww1.microchip.com/downloads/en/DeviceDoc/40001819B.pdf [Accessed on:

3/23/2019]

[4]Author:Steve Fuller, Date updated: 4/17/2019, https://www.statista.com/topics/1639/music/ [Accessed on: 4/15/2019] [5] ENEL 369 LAB Documents, “University of Calgary”, 2019

[6]Image:  https://www.guru99.com/testing-methodology.html#4 [Accessed on: 4/17/2019]

7.2. User Manual

Are you ready to become a professional brass instrument player and take your skills to the next

level? Well, look no further as “Brass2Go” is designed for music fanatics like you!

Why was it created?

Brass2Go was inspired by the highly competitive music industry and the lack of a portable learning tool to practice brass instruments. Over 90% of music professionals are considered “unknown” by the public and only less than 1% of all music professionals showcase their talent to people like you and I. Why are we restricted to certain musicians and limiting our creativity?

Thus, Brass2Go is a learning tool for the 90% we would like to cater to and help them improve their skills to compete with the successful 1% musicians and share their talents with the world!

What is Brass2Go and how does it help you improve your skills?

Brass2Go is a portable device that allows brass musicians to practice their instruments and compositions on-the-go. The device, powered by a PIC16F1778 microcontroller, plays an audio (.wav) file from a microSD card and the user plays the finger sequence along with the song. If the user plays the note incorrectly (i.e. the wrong note and/or at the wrong time) a red LED will light up. In contrast, a green LED will light up if the note is played correctly. Once the file is finished playing, an LCD screen will display the number of notes played incorrectly, and the percent of notes played correctly (e.g. 5/26 wrong -- 80.7% correct). During playback, the user can pause and play.

Please follow the user-friendly instructions on the next page to play!

Play Brass2Go in only 4 easy steps!

STEP 1                            Press the switch to turn on your

Brass2Go

STEP 2                            Press the first valve to start your desired song stored in the SD Card

STEP 3                            Start your magic and play the trumpet as soon as you hear the song! (Can also PAUSE/PLAY)

STEP 4                            The LCD will output your results at the end of the song.